TAP — Ruckus SmartZone syslog to Coralogix

Device configuration only. SmartZone and its APs must send traditional syslog (RFC3164) over UDP to the TAP collector. The controller can do TCP; this template does not. Use UDP.

1. Where this device must send

The collector is already a separate Compose service. This guide only covers the device. Point the device at the Compose host IP, never 127.0.0.1.

| Setting | Value |
| --- | --- |
| Transport | UDP (not TCP) |
| Port | 5515 |
| Format | RFC3164 / BSD syslog (controller default) |
| Coralogix application | ruckus |
| Coralogix subsystem | access-point |

Replace <COMPOSE_HOST_IP> with the address this device can route to. If the host port in .env was changed, use that port, not the default below.

2. Two places, same destination

SmartZone has a controller syslog and a per-zone AP external syslog. Point both at the same host, port 5515, UDP. Controller-only syslog misses AP client events.

| What | Where | Sends |
| --- | --- | --- |
| Controller syslog | System → General Settings → Syslog | Controller events |
| AP external syslog | Zone edit → Syslog, or Services → AP External Syslog | AP client join / auth / disconnect |

3. Controller syslog

SmartZone web UI → System → General Settings → Syslog.

Enable logging to remote syslog server.

Primary server address = <COMPOSE_HOST_IP>.

Port = 5515. Protocol = UDP. Do not leave this on TCP.

Click Ping Syslog Server. Expect Success before you save.

Facility: leave default, or 0–7 if your site standard requires a facility.

Filter severity: Informational is enough for client join/auth. Debug floods the link.

Event filter: All events for a first integration. Later you can drop client association/disassociation if volume is too high.

OK / Apply.

Figure 1: System > General Settings > Syslog — enable checkbox, primary server, port, protocol UDP, facilities and event filter. Example values shown; for TAP use <COMPOSE_HOST_IP> and port 5515.

Menu names drift by SmartZone version (SZ100 / vSZ / SZ300). If General Settings → Syslog is not there, search the UI for Syslog. The fields that matter are address, port 5515, and protocol UDP.

4. AP external syslog (required for client events)

Services → AP External Syslog Server (wording may be AP External Syslog).

Create or edit a profile: same <COMPOSE_HOST_IP>, port 5515, UDP. Send Logs = All Logs for the first test.

Access Points → select the AP zone → Edit → Syslog.

Enable external syslog server for APs. Select that profile. OK.

Figure 2: Zone (Configure Group) > Configuration > Syslog Options — enable toggle, primary server, port, protocol, Send Logs. This is the zone-level panel that forwards AP client events.

Figure 3: Same panel with the Protocol fields (boxed) — set primary to UDP for this template.

A zone that does not have this toggle on will not forward AP client syslog, even if the controller syslog ping succeeded.

5. What a good line looks like

You do not type these. They are the shapes the controller actually emits, so you can recognise a successful send:

User[<mac>] fails authentication in WLAN[<wlan>] from AP[<ap>]
User[<mac>] joins WLAN[<wlan>] at AP[<ap>]
Client [<user>@<ip>@<mac>] joined WLAN [<wlan>] from AP [<ap>@<mac>]

6. Check from SmartZone

Ping Syslog Server in the syslog page returns Success.

Join a test client to a WLAN in the zone you edited. A join or auth-fail line should appear under application ruckus / subsystem access-point.

If only controller alarms arrive and client lines do not, section 4 is not applied on that zone.

7. If nothing arrives

| What you see | Fix on SmartZone |
| --- | --- |
| Ping Syslog Server fails | Wrong IP, or UDP 5515 blocked between the controller and the Compose host. |
| Protocol is TCP | Switch the primary server to UDP. This collector listens on UDP only. |
| Ping succeeds, no client logs | AP external syslog is not enabled on the zone. |
| Logs appear, then stop | Event filter excludes association, or severity is above Informational. |
| Collector parse error | Do not change the collector to RFC5424. SmartZone remote syslog is BSD/RFC3164. |

Source: Ruckus SmartZone administrator guide (System → General Settings → Syslog; protocol TCP or UDP) and SmartZone AP external syslog (zone → Syslog). TAP collector listens on UDP 5515, RFC3164.